/*

============================================================================

VAWT ROADSIDE KINETIC ENERGY HARVESTER — MONITOR & STREETLIGHT CONTROLLER

(MODIFIED VERSION — LDR removed, light is now driven purely by generated

voltage from the 3 dynamo motors)

============================================================================

Project idea:

A Vertical Axis Wind Turbine (VAWT) sits on the road divider. Airflow

(waste kinetic energy) pushed out by passing vehicles spins the turbine.

The turbine shaft turns 3 small DC dynamo/generator motors. The power

generated charges a battery bank, which is used to run highway lights

/ small loads.

What changed in this version:

- LDR sensor and all ambient-light logic have been REMOVED.

- The streetlight LED now turns on purely based on whether the 3

combined dynamo motors are actually generating voltage. As soon as

you rotate the motor shaft, the voltage sensor sees a rise in

voltage, and the LED blinks / lights up — no darkness required.

What this sketch does now (bench-demo logic):

1. Reads generator output through an ACS712 current sensor and a

voltage-divider voltage sensor -> computes instantaneous power.

2. Reads shaft speed with a hall-effect sensor -> computes RPM

(kept purely for telemetry / dashboard display, not for the

light decision anymore).

3. Reads temperature/humidity with a DHT11 for environmental logging.

4. Streetlight (LED / relay) logic — now voltage-driven:

- If generated voltage is BELOW threshold -> light OFF

(motor isn't spinning / not generating enough)

- The MOMENT voltage crosses the threshold (motor just started

spinning) -> blink a few times (visual proof for evaluators

that "rotation = generation = light")

- While voltage STAYS above threshold -> light ON steady

- If voltage drops back below threshold -> light OFF

5. Streams all readings once a second as one JSON line over Serial,

which the companion dashboard.html reads live via the Web Serial

API (Chrome/Edge) and turns into gauges + a live chart.

Hardware used:

- ACS712 current sensor        -> A0

- Voltage sensor module        -> A1   (this now drives the light logic)

- DHT11 temperature/humidity   -> D2

- Hall effect sensor (RPM)     -> D3   (telemetry only, no wiring change)

- Streetlight LED / relay IN   -> D9   (PWM capable, used for blinking)

- Status LED (optional)        -> D13  (onboard LED mirrors light state)

NOTE ON THE "3 DYNAMO MOTORS":

The 3 generator motors are wired together (in parallel, after individual

rectifier diodes/bridge rectifiers so they don't fight each other) into

ONE combined DC output. THAT combined output is what feeds the ACS712 /

voltage sensor / battery. You do not wire 3 separate sensors — you

combine the generators first, then measure the combined result. This

is standard practice for multi-generator harvesting setups.

Libraries needed (Arduino IDE -> Library Manager):

- "DHT sensor library" by Adafruit  (+ "Adafruit Unified Sensor")

============================================================================

*/

#include <DHT.h>

// ---------------------------- PIN DEFINITIONS ------------------------------

#define DHT_PIN            2

#define DHT_TYPE           DHT11

#define HALL_PIN           3      // interrupt-capable pin (D2/D3 on Uno)

#define ACS712_PIN         A0

#define VOLTAGE_PIN        A1

#define STREETLIGHT_PIN    9      // PWM pin -> LED (through resistor) or relay

#define STATUS_LED_PIN     13     // onboard LED mirrors streetlight state

// ---------------------------- CALIBRATION ----------------------------------

// ACS712 variant sensitivity: 5A model = 185 mV/A, 20A = 100 mV/A, 30A = 66 mV/A

#define ACS712_MV_PER_AMP     185.0

#define ACS712_ZERO_VOLTAGE   2.500   // sensor output at 0 A (measure yours, ~2.5V typical)

// Voltage sensor module divider ratio. Common cheap module: (R1+R2)/R2 = 5.0

// Recalibrate: measure real motor voltage with a multimeter, compare to

// raw reading, and adjust this constant until they match.

#define VOLTAGE_DIVIDER_RATIO 5.0

#define ARDUINO_VREF           5.0

#define ADC_RESOLUTION         1023.0

// Hall sensor: how many magnet pulses per one full shaft revolution

#define PULSES_PER_REV         1

// ---- STREETLIGHT TRIGGER (now based on generated voltage, not RPM/LDR) ----

// Minimum combined dynamo voltage that counts as "motor is generating /

// shaft is being rotated". Hand-spun small DC motors used as generators

// typically put out a couple of volts even at slow hand-crank speeds, so

// start with a low value and raise it once you've measured your own motors.

#define VOLTAGE_SPIN_THRESHOLD   1.0    // volts

// How many quick blinks to show when generated voltage crosses the threshold

#define STARTUP_BLINK_COUNT    4

#define STARTUP_BLINK_MS       150

// How often we push a JSON reading to Serial (ms)

#define REPORT_INTERVAL_MS     1000

// How often we recompute RPM from pulse count (ms)

#define RPM_WINDOW_MS           1000

// ---------------------------- GLOBAL STATE ----------------------------------

DHT dht(DHT_PIN, DHT_TYPE);

volatile unsigned long pulseCount = 0;

unsigned long lastRpmCalcTime = 0;

float currentRPM = 0;

unsigned long lastReportTime = 0;

bool wasGenerating = false;

bool streetlightOn = false;

// ---------------------------- ISR --------------------------------------

void hallISR() {

pulseCount++;

}

// ---------------------------- SETUP --------------------------------------

void setup() {

Serial.begin(9600);

pinMode(HALL_PIN, INPUT_PULLUP);

pinMode(STREETLIGHT_PIN, OUTPUT);

pinMode(STATUS_LED_PIN, OUTPUT);

attachInterrupt(digitalPinToInterrupt(HALL_PIN), hallISR, FALLING);

dht.begin();

digitalWrite(STREETLIGHT_PIN, LOW);

digitalWrite(STATUS_LED_PIN, LOW);

lastRpmCalcTime = millis();

lastReportTime = millis();

Serial.println(F("{\"status\":\"VAWT Monitor booted (voltage-triggered light)\"}"));

}

// ---------------------------- HELPERS --------------------------------------

float readCurrentAmps() {

int raw = analogRead(ACS712_PIN);

float voltage = (raw / ADC_RESOLUTION) * ARDUINO_VREF;

float amps = (voltage - ACS712_ZERO_VOLTAGE) / (ACS712_MV_PER_AMP / 1000.0);

if (abs(amps) < 0.05) amps = 0.0;   // small dead-band to hide sensor noise

return amps;

}

float readVoltage() {

int raw = analogRead(VOLTAGE_PIN);

float voltage = (raw / ADC_RESOLUTION) * ARDUINO_VREF * VOLTAGE_DIVIDER_RATIO;

return voltage;

}

void updateRPM() {

unsigned long now = millis();

if (now - lastRpmCalcTime >= RPM_WINDOW_MS) {

noInterrupts();

unsigned long pulses = pulseCount;

pulseCount = 0;

interrupts();

float minutes = (now - lastRpmCalcTime) / 60000.0;

currentRPM = (pulses / (float)PULSES_PER_REV) / minutes;

lastRpmCalcTime = now;

}

}

// Blocking startup blink sequence — proves "rotation just began -> light reacts"

void playStartupBlink() {

for (int i = 0; i < STARTUP_BLINK_COUNT; i++) {

digitalWrite(STREETLIGHT_PIN, HIGH);

digitalWrite(STATUS_LED_PIN, HIGH);

delay(STARTUP_BLINK_MS);

digitalWrite(STREETLIGHT_PIN, LOW);

digitalWrite(STATUS_LED_PIN, LOW);

delay(STARTUP_BLINK_MS);

}

}

// Core decision logic — light now reacts purely to generated voltage

void updateStreetlight(float volts) {

bool generating = volts > VOLTAGE_SPIN_THRESHOLD;

if (generating && !wasGenerating) {

// Voltage just crossed the threshold -> motor just started spinning

playStartupBlink();

}

wasGenerating = generating;

streetlightOn = generating;

digitalWrite(STREETLIGHT_PIN, streetlightOn ? HIGH : LOW);

digitalWrite(STATUS_LED_PIN, streetlightOn ? HIGH : LOW);

}

// ---------------------------- MAIN LOOP --------------------------------------

void loop() {

updateRPM();

float amps  = readCurrentAmps();

float volts = readVoltage();

updateStreetlight(volts);   // checked every loop, not just once a second, for a snappy response

unsigned long now = millis();

if (now - lastReportTime >= REPORT_INTERVAL_MS) {

lastReportTime = now;

float watts = amps * volts;

float temp  = dht.readTemperature();

float hum   = dht.readHumidity();

// Guard against DHT11 read failures (common, non-fatal)

if (isnan(temp)) temp = 0;

if (isnan(hum))  hum = 0;

// Single-line JSON -> easy for the HTML dashboard to parse

Serial.print(F("{"));

Serial.print(F("\"rpm\":"));       Serial.print(currentRPM, 1);

Serial.print(F(",\"voltage\":"));  Serial.print(volts, 2);

Serial.print(F(",\"current\":"));  Serial.print(amps, 3);

Serial.print(F(",\"power\":"));    Serial.print(watts, 2);

Serial.print(F(",\"temp\":"));     Serial.print(temp, 1);

Serial.print(F(",\"humidity\":"));Serial.print(hum, 1);

Serial.print(F(",\"light\":\""));  Serial.print(streetlightOn ? "ON" : "OFF"); Serial.print(F("\""));

Serial.println(F("}"));

}

}